| <Image_288pxW_MainModel/> | <Label_Simulationof/> <Model_Name/> <Label_Date/>: <Published_Date/> <Label_Designer/>: <Designer_Name/> <Label_StudyName/>: <Study_Name/> <Label_StudyType/>: <Study_Type/>. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 9 Study Results 10 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: AquaSentinel_Pier_Bracket_Simulation Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Pier bracket weldment (AS3-101), welded for FEA; Solid Body; Mass:12.6766 kg Volume:0.00157924 m^3 Density:8,027 kg/m^3 Weight:124.23 N; C:\Users\dell\.gemini\antigravity\scratch\AquaSentinel_AAKRUTI\FDC_Submission\3_Simulation\AquaSentinel_Pier_Bracket_Simulation.SLDPRT Oct 5 13:53:40 2026.

# Study Properties Study name; Bracket pole reaction. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\dell\.gemini\antigravity\scratch\AquaSentinel_AAKRUTI\FDC_Submission\3_Simulation).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI Type 316L stainless steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 1.7e+08 N/m^2. Tensile strength:; 4.85e+08 N/m^2. Elastic modulus:; 2e+11 N/m^2. Poisson's ratio:; 0.265. Mass density:; 8,027 kg/m^3. Shear modulus:; 8.2e+10 N/m^2. Thermal expansion coefficient:; 1.65e-05 /Kelvin.; SolidBody 1(Pier bracket weldment (AS3-101), welded for FEA)(AquaSentinel_Pier_Bracket_Simulation). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -430.003; 0.0012731; 161.004; 459.157. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 215 N.. Force-2; Entities:; 1 face(s), 1 plane(s). Reference:; Front Plane. Type:; Apply force. Values:; ---, ---, -80.5 N.. Force-3; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 215 N.. Force-4; Entities:; 1 face(s), 1 plane(s). Reference:; Front Plane. Type:; Apply force. Values:; ---, ---, -80.5 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 4 mm. Tolerance; 0.2 mm. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 268389. Total Elements; 174305. Maximum Aspect Ratio; 16.824. % of elements with Aspect Ratio < 3; 99.8. % of elements with Aspect Ratio > 10; 0.0625. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:07. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -430.003; 0.0012731; 161.004; 459.157. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 7.098e+00 N/m^2 Node: 108; 3.651e+06 N/m^2 Node: 161657. AquaSentinel_Pier_Bracket_Simulation-Bracket pole reaction-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00 mm Node: 19; 4.110e-03 mm Node: 17749. AquaSentinel_Pier_Bracket_Simulation-Bracket pole reaction-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.956e-11 Element: 172385; 1.090e-05 Element: 1160. AquaSentinel_Pier_Bracket_Simulation-Bracket pole reaction-Strain-Strain1. Name; Type; Min; Max. Stress2; VON: von Mises Stress; 7.098e+00 N/m^2 Node: 108; 3.651e+06 N/m^2 Node: 161657. AquaSentinel_Pier_Bracket_Simulation-Bracket pole reaction-Stress-Stress2. Name; Type; Min; Max. Displacement2; URES: Resultant Displacement; 0.000e+00 am Node: 19; 4.110e+04 am Node: 17749. AquaSentinel_Pier_Bracket_Simulation-Bracket pole reaction-Displacement-Displacement2.

# Conclusion