|  | Simulation of AquaSentinel_Guide_Pole_Simulation Date: 05 October 2026 Designer: Solidworks Study name: Pole flood and debris Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 4 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Contact Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: AquaSentinel_Guide_Pole_Simulation Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line1; Solid Body; Mass:16.558 kg Volume:0.00206278 m^3 Density:8,027 kg/m^3 Weight:162.268 N; C:\Users\dell\.gemini\antigravity\scratch\AquaSentinel_AAKRUTI\FDC_Submission\3_Simulation\AquaSentinel_Guide_Pole_Simulation.SLDPRT Oct 5 13:46:07 2026.

# Study Properties Study name; Pole flood and debris. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\dell\.gemini\antigravity\scratch\AquaSentinel_AAKRUTI\FDC_Submission\3_Simulation).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; AISI Type 316L stainless steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 1.7e+08 N/m^2. Tensile strength:; 4.85e+08 N/m^2. Elastic modulus:; 2e+11 N/m^2. Poisson's ratio:; 0.265. Mass density:; 8,027 kg/m^3. Shear modulus:; 8.2e+10 N/m^2. Thermal expansion coefficient:; 1.65e-05 /Kelvin.; SolidBody 1(Split Line1)(AquaSentinel_Guide_Pole_Simulation). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -871.283; 0.0621969; -0.0211442; 871.283. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 645.256 N.. Force-2; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 104.111 N.. Force-3; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 104.111 N.. Force-4; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 8.954 N.. Force-5; Entities:; 1 face(s), 1 plane(s). Reference:; Right Plane. Type:; Apply force. Values:; ---, ---, 8.954 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 6 mm. Tolerance; 0.3 mm. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 179650. Total Elements; 89812. Maximum Aspect Ratio; 4.8992. % of elements with Aspect Ratio < 3; 99.8. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:36. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -871.283; 0.0621969; -0.0211442; 871.283. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.296e+01 N/m^2 Node: 78267; 3.832e+07 N/m^2 Node: 103421. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00 mm Node: 9485; 1.255e+00 mm Node: 54763. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 1.036e-10 Element: 7690; 1.323e-04 Element: 38188. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Strain-Strain1. Name; Type; Min; Max. Stress2; VON: von Mises Stress; 1.296e+01 N/m^2 Node: 78267; 3.832e+07 N/m^2 Node: 103421. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Stress-Stress2. Name; Type; Min; Max. Displacement2; URES: Resultant Displacement; 0.000e+00 am Node: 9485; 1.255e+07 am Node: 54763. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Displacement-Displacement2. Name; Type; Min; Max. Displacement3; UX: X Displacement; -2.085e+03am Node: 550; 1.255e+07 am Node: 54763. AquaSentinel_Guide_Pole_Simulation-Pole flood and debris-Displacement-Displacement3.

# Conclusion